NEURONEXUS TEST PAPER 01

Title

IoT-Based Smart Irrigation System Using Soil Moisture Monitoring and Machine Learning

Authors

Deekshith Kumar, Research Team
Department of Electronics and Communication Engineering

Abstract

Agriculture requires efficient water management because excessive irrigation can increase water consumption and negatively affect crop productivity. This paper presents an IoT-based smart irrigation system that monitors soil moisture and automatically controls irrigation based on sensor measurements. The proposed system uses an ESP32 microcontroller, soil moisture sensors, and a water pump. Sensor readings are transmitted to a cloud platform for monitoring. A machine learning model is additionally used to predict irrigation requirements based on historical sensor data. The system was tested under different soil moisture conditions and achieved an accuracy of 94.2%.

Keywords

IoT, Smart Irrigation, Soil Moisture, ESP32, Machine Learning, Agriculture

1. Introduction

Water management is an important problem in modern agriculture. Traditional irrigation methods often provide water without considering the actual moisture condition of soil. IoT-based systems can continuously monitor environmental parameters and help automate irrigation decisions.

The proposed system uses an ESP32 controller connected to soil moisture sensors. When the measured moisture level falls below a predefined threshold, the controller activates a water pump. The system also stores sensor readings for further analysis.

Several researchers have investigated IoT-based irrigation systems [1], [2], and [3]. Machine learning has also been applied to agricultural prediction problems [5].

2. Related Work

Previous studies have investigated sensor-based irrigation and automated agricultural monitoring. Smith et al. [1] proposed an IoT irrigation system using wireless sensors. Kumar et al. [2] developed an automated irrigation controller based on soil moisture measurements.

Another study presented a machine learning approach for predicting irrigation requirements [3].

These systems demonstrate the usefulness of IoT technologies in agriculture. However, many systems depend on fixed threshold values and do not consider historical sensor patterns.

3. Research Gap

Existing irrigation systems commonly use fixed soil moisture thresholds. Such systems may not adapt effectively to changing environmental conditions.

The research gap addressed in this work is the need for a system that combines real-time soil moisture monitoring with machine learning-based irrigation prediction.

4. Methodology

The proposed system consists of an ESP32 microcontroller, soil moisture sensors, a relay module, and a water pump.

The soil moisture sensor measures the moisture level of the soil. The ESP32 processes the sensor readings and determines whether irrigation is required.

The machine learning model uses sensor readings as input and predicts whether irrigation should be activated.

The model was trained using agricultural sensor data.

5. Dataset

The dataset contains soil moisture and environmental measurements collected from an experimental agricultural setup.

The dataset contains 5,000 sensor records.

The data was divided into training and testing sets.

6. Experimental Setup

The prototype was tested using different soil moisture conditions.

The ESP32 was connected to the soil moisture sensor and relay-controlled water pump.

The machine learning model was evaluated using accuracy as the primary metric.

However, the exact train-test split, model parameters, baseline methods, and hardware test duration were not documented.

7. Results

The proposed machine learning model achieved an accuracy of 94.2%.

The IoT system successfully activated the pump when the soil moisture level decreased below the configured threshold.

Table 1. System Results

| Condition | Pump Status | Soil Moisture |
| --- | --- | --- |
| Dry Soil | ON | 25% |
| Moderate Soil | OFF | 55% |
| Wet Soil | OFF | 78% |

8. Discussion

The results show that the proposed system can automatically control irrigation based on soil moisture conditions.

The machine learning model provides useful predictions for irrigation requirements.

However, additional experiments with different crops, soil types, weather conditions, and longer testing periods are required.

9. Limitations

The current system was tested in a controlled environment.

The dataset was collected from a limited experimental setup.

The system was not evaluated across multiple agricultural locations.

The model was evaluated primarily using accuracy, without detailed comparison against baseline machine learning algorithms.

10. Conclusion

This paper presented an IoT-based smart irrigation system using soil moisture monitoring and machine learning.

The proposed approach combines automated irrigation control with predictive analysis.

Future work will investigate larger datasets, additional environmental parameters, and evaluation across different agricultural environments.

References

[1] Smith, J., "IoT Based Smart Irrigation System," International Journal of Agricultural Technology, 2022.

[2] Kumar, R., "Automated Soil Moisture Monitoring for Agriculture," Journal of Smart Agriculture, 2023.

[3] Patel, A., "Machine Learning for Irrigation Prediction," International Conference on Agricultural Computing, 2024.

[5] Rao, P., "Intelligent Agricultural Monitoring Using IoT," Journal of IoT Applications, 2023.

[5] Rao, P., "Intelligent Agricultural Monitoring Using IoT," Journal of IoT Applications, 2023.